	HowTo-> Paddle shifters from Mercedes on a E38 and yes it works! (long)
Author: homerraas as Edwin™ [image: image1.jpg]


[image: image2.jpg]


: 23107 posts
Posted on: 2009-01-07 11:41:12 from 83.80.150.248 (535096F8.cable.casema.nl)       
	




Take the wheel out of the car




Remove the Airbag




Make a template of the paddle 







Mark the template on the wheel




And start to cut?!?!?







Drill 2 holes, 1 for the cable and one for the screw to hold the paddle in place




Test fit




Bring the wires in the center 




Test fit and adjust to make it straight













Remove the gearbox knob and surrounding trim




You need the 2 middle wires
Blue-white= up shift
blue-yellow= down shift




Remove the old slipring (no contacts in upper left connector)







Install the new one




Run the wires from the slipring to the gearbox and wire them up




Put the wheel back on and go and drive her




The paddles are $100 from Mercedes (AMG paddles) and the slipring and 2 connectors were $110

Do you like this? Did you learn from it? Why not make a contribution to support us and keep this board alive!

[image: image25.jpg]BIMMERB0aRD cov)





Edwin
The Netherlands
735i 3.5 V8 Euro 2001 High Executive
Steptronic+ Switch-Tronic, Leather, Leather Dash, Leather Center console, Leather lower dash
Xenon 8k + Angel Eyes, Xenon 6k Fogs, Full 750 lighting (inside)
Double glazing, 16:9 + MK IV (V31 in Dutch)
DSP (retro), Bass link, Comfort seats, Front/rear seat heating, Massage function
Tire pressure control, Intensive window cleaner
DVB-T Digital TV (in motion), DVD/MP3 changer, Siemens BIT II phone system, Style 95 19" rims
Strong Strut, Eibach pro kit (front only), Bilstein B6 (HD)(front and rear),Alpina leather steering wheel
Zimmerman brakes,SES voice control, Hydraulic trunk open/close 





The 7 Series E38 Forum | Message Thread:

· HowTo-> Paddle shifters from Mercedes on a E38 and yes it works! (long) (pic) (link) (imp) - Edwin™[image: image26.jpg]


[image: image27.jpg]


 2009-01-07 11:41:12 (17336 views) 

· ??? - jls06x 2009-01-09 17:53:54 (393 views) 

· Man this really make's me want to do that steptronic retrofit. (nt) - Lexicon 740iL[image: image28.jpg]


 2009-01-08 17:12:49 (504 views) 

· +1 not quite as easy on my car either... (nt) - 04sshd[image: image29.jpg]


 2009-01-08 17:49:31 (488 views) 

· Do they get in the way a bit when turning the steering wheel? How did you know y - Steve R[image: image30.jpg]


 2009-01-08 09:59:09 (405 views) 

· Nope don't get in the way and if you have steptronic it will work 100% (nt) - Edwin™[image: image31.jpg]


[image: image32.jpg]


 2009-01-08 10:13:59 (412 views) 

· Aren't the paddles just a remote switch for the Steptronic shifter lever? (nt) - Roy In GA[image: image33.jpg]


 2009-01-08 10:10:05 (551 views) 

· Yes Roy it's both now! but when doing 150mph I want my hands on the wheel :-) (nt) - Edwin™[image: image34.jpg]


[image: image35.jpg]


 2009-01-08 10:14:44 (372 views) 

· You shift gears at 150..........Holy crap!! (nt) - beechy[image: image36.jpg]


 2009-01-08 10:23:45 (352 views) 

· In fact if you look at my video, Yes it does shift to 5 @150MPH! (nt) - Edwin™[image: image37.jpg]


[image: image38.jpg]


 2009-01-08 10:56:46 (403 views) 

· Yep...like I said buddy.......Holy Crap!!.........;o) (nt) - beechy[image: image39.jpg]


 2009-01-08 11:05:50 (313 views) 

· E, You are one crazy, knowledgable SOB! Damn nice work! (nt) - IBHenry®[image: image40.jpg]


 2009-01-07 22:06:15 (311 views) 

· Nice work! Sport wheel + paddle shifter = best of both worlds... (link) - sealbeach740[image: image41.jpg]


 2009-01-07 19:20:30 (459 views) 

· I thought this looked familiar... - M.Wong[image: image42.jpg]


[image: image43.jpg]


 2009-01-07 21:43:17 (339 views) 

· Very cool E. (nt) - 740ilDuke[image: image44.jpg]


[image: image45.jpg]


 2009-01-07 18:55:11 (274 views) 

· Very innovative! (nt) - DavidC[image: image46.jpg]


[image: image47.jpg]


 2009-01-07 15:14:57 (289 views) 

· Anyone thinks it possible with the non-sport wheel? - eaglecomm[image: image48.jpg]


[image: image49.jpg]


 2009-01-07 14:53:57 (328 views) 

· Nope paddles are to big to fit the 4 spoke and no you will loose heating (nt) - Edwin™[image: image50.jpg]


[image: image51.jpg]


 2009-01-07 15:02:02 (341 views) 

· If you love the 4 spoke just get one of these (if you can find them) :-) (link) - Edwin™[image: image52.jpg]


[image: image53.jpg]


 2009-01-07 15:04:34 (561 views) 

· do you still have yours? (nt) - 04sshd[image: image54.jpg]


 2009-01-07 19:35:54 (282 views) 

· Yep, My Alpina wheel is going no were! I love it (nt) - Edwin™[image: image55.jpg]


[image: image56.jpg]


 2009-01-08 07:02:43 (311 views) 

· 
No.. I love the heated wheel. :'( If the 3-spoke had it.. woohoo! Alas.. no.
 (nt) - eaglecomm[image: image58.jpg]


[image: image59.jpg]


 2009-01-07 15:12:31 (334 views) 

· Any part numbers? (nt) - ksmith38a[image: image60.jpg]


 2009-01-07 14:43:59 (339 views) 

· For the paddles? (nt) - Edwin™[image: image61.jpg]


[image: image62.jpg]


 2009-01-07 15:02:21 (391 views) 

· yes and the slipring if you have it handy. (nt) - 740isport 2009-01-07 15:36:55 (379 views) 

· Paddles off Ebay and slipring look it up on realoem, it depends on the car what - Edwin™[image: image63.jpg]


[image: image64.jpg]


 2009-01-07 16:05:05 (528 views) 

· I think this is the Mercedes paddle part # - eaglecomm[image: image65.jpg]


[image: image66.jpg]


 2009-01-07 15:51:07 (503 views) 

· Sorry.. wrong part # for the paddles. Here it is: 001 A 221 267 00 46 - eaglecomm[image: image67.jpg]


[image: image68.jpg]


 2009-01-07 22:20:51 (960 views) 

· I love it, wish I had Tiptronic tranny or is it call steptronic with BMW (nt) - rg0115[image: image69.jpg]


 2009-01-07 12:45:57 (307 views) 

· Check this out, Robert. I think that's all the hardware you need. Eaglecomm did (link) - Wiggi[image: image70.jpg]


[image: image71.jpg]


 2009-01-07 12:57:50 (490 views) 

· you would need that shifter, the bezel, and the steptronic trans computer (nt) - 04sshd[image: image72.jpg]


 2009-01-07 12:59:17 (286 views) 

· so my exsisting tranny could support this setup. How much would the entie mod - rg0115[image: image73.jpg]


 2009-01-07 14:18:32 (326 views) 

· Check with Scubaa Steve for the trans computer - eaglecomm[image: image74.jpg]


[image: image75.jpg]


 2009-01-07 14:49:33 (479 views) 

· YGM (nt) - Shled[image: image76.jpg]


[image: image77.jpg]


 2009-01-08 08:58:08 (270 views) 

· No email in the inbox or junk mail.. - eaglecomm[image: image78.jpg]


[image: image79.jpg]


 2009-01-08 09:27:02 (290 views) 

· Doesn't he already have steptronic in his sport? Then wouldn't this.. - sdmike[image: image80.jpg]


[image: image81.jpg]


 2009-01-07 23:02:49 (292 views) 

· Isn't rg0115 a 740iL with sport blackout, not an i sport? - 740isport 2009-01-07 23:52:25 (305 views) 

· no man. Robert is driving a black on black sport (nt) - sdmike[image: image82.jpg]


[image: image83.jpg]


 2009-01-07 23:56:24 (283 views) 

· Then why does his sig show 740iL? (nt) - 740isport 2009-01-07 23:57:21 (315 views) 

· my bad. Thought he had an i, not il (nt) - sdmike[image: image84.jpg]


[image: image85.jpg]


 2009-01-07 23:58:12 (294 views) 

· Mike, i have an IL. No steptronic unfortunately (nt) - rg0115[image: image86.jpg]


 2009-01-08 00:31:00 (328 views) 

· The shifter bezel that removes the M/A switch and shows - 740isport 2009-01-07 15:38:42 (339 views) 

· Ah.. gotcha.. I was assuming that was included in the sale in the FS forum - eaglecomm[image: image87.jpg]


[image: image88.jpg]


 2009-01-07 15:49:51 (309 views) 

· Thanks for sharing. Considering I need a sport wheel plus the other cost - rg0115[image: image89.jpg]


 2009-01-07 15:24:27 (283 views) 

· Whimp! ;-) (nt) - Edwin™[image: image90.jpg]


[image: image91.jpg]


 2009-01-07 16:06:15 (277 views) 

· You hurt my feelings. I am not a whimp, just don't have the extra cash - rg0115[image: image92.jpg]


 2009-01-07 16:25:23 (280 views) 

· Dont you already have steptronic? Would you still need the computer? (nt) - sdmike[image: image93.jpg]


[image: image94.jpg]


 2009-01-07 23:01:31 (285 views) 

· He's an iL sport so no steptronic without trans computer. (nt) - 740isport 2009-01-07 23:56:15 (300 views) 

· ahhh. .I thought he was driving an i, not an il. I just remember the.. - sdmike[image: image95.jpg]


[image: image96.jpg]


 2009-01-07 23:57:42 (272 views) 

· 
LOL and we are knocking back and forth on two lines! 
(nt) - 740isport 2009-01-07 23:58:34 (296 views) 

· that's ok. Monitor ink is cheap. lol (nt) - sdmike[image: image98.jpg]


[image: image99.jpg]


 2009-01-08 00:02:03 (264 views) 

· Supercharger, supercharger.... (nt) - Wiggi[image: image100.jpg]


[image: image101.jpg]


 2009-01-07 16:39:45 (282 views) 

· Chicken ;-) J/k (nt) - Wiggi[image: image102.jpg]


[image: image103.jpg]


 2009-01-07 16:01:51 (265 views) 

· do I hear a retrofit comming (nt) - 04sshd[image: image104.jpg]


 2009-01-07 12:46:37 (293 views) 

· by the looks of this I think it can be done with a single stage airbag too... - 04sshd[image: image105.jpg]


 2009-01-07 12:36:28 (334 views) 

· I thought 2 stage SRS and Steptrotric goes together so , stage 1 SRS - bmw635[image: image106.jpg]


 2009-01-07 12:53:39 (346 views) 

· Edwin is running the steptronic wires through the wheel heater plug. - 04sshd[image: image107.jpg]


 2009-01-07 12:58:21 (456 views) 

· Yep you got it all Colin! Black goes to the ground of the airbag (nt) - Edwin™[image: image108.jpg]


[image: image109.jpg]


 2009-01-07 13:35:27 (346 views) 

· Good to know, if I could get a steptronic trans computer I would do this mod! - 04sshd[image: image110.jpg]


 2009-01-07 17:44:28 (307 views) 

· Write the seller and see if this one would work.... (link) - eaglecomm[image: image111.jpg]


[image: image112.jpg]


 2009-01-08 14:48:27 (334 views) 

· I don't know, we all have steptronic here so never looked into it (nt) - Edwin™[image: image113.jpg]


[image: image114.jpg]


 2009-01-08 07:04:24 (307 views) 

· Did you take off the Alpina wheel E? I love the Merk as well! - bghark[image: image115.jpg]


 2009-01-07 12:11:36 (278 views) 

· Nope I love my Alpina wheel, this was a wheel of a dutch member (nt) - Edwin™[image: image116.jpg]


[image: image117.jpg]


 2009-01-07 13:36:17 (319 views) 

· I love that wheel as well. The button shift is just too cool. (nt) - bghark[image: image118.jpg]


 2009-01-07 14:21:09 (306 views) 

· Very nice and looks factory! - gmblack3[image: image119.jpg]


 2009-01-07 12:05:26 (377 views) 

· I wish that was possible! btw downsize your sig pic a bit! :-) (nt) - Edwin™[image: image120.jpg]


[image: image121.jpg]


 2009-01-07 13:37:16 (315 views) 



This thread is closed to new posts.
